Light-Sport Aircraft Maintenance and Certification Requirements

|  | Ultralights, part 103--254 pounds EW | ELSA under §21.191 (i)-- 1,320 pounds MTOW | SLSA under §21.190--1,320 pounds MTOW | Amateur-built under §21.191 (g)--No MTOW | Primary aircraft under §21.24--2,700 pounds MTOW | Standard aircraft under §21.21--No MTOW |
| --- | --- | --- | --- | --- | --- | --- |
| Regist-ration "N" number | None | Yes | Yes | Yes | Yes | Yes |
| Airworth-iness Certificate | None | Yes | Yes | Yes | Yes | Yes |
| Operating privileges and limits (may be restricted by pilot certificate or aircraft operating limitations) | Day VFR Uncong-ested areas only Class A, B, C, D, and surface-based E airspace; ATC permission required (no equipment required) Class E and G airspace No passengers Flight training (under part 103 two-place training exemption) Towing (under part 103 towing exemption) | Day/night VFR/IFR Congested and uncongested areas Class A, B, C, D, E and G airspace; (with part 91 equipment) One passenger Limited flight training (5 years under §91.319 (e)(2)) Towing existing fleet (under §91.319 (e)(1)) | Day/night VFR/IFR Congested and uncongested areas Class A, B, C, D, E and G airspace; (with part 91 equipment) One passenger Flight training and rental Towing | Day/night VFR/IFR Congested and uncongested areas Class A, B, C, D, E and G airspace; (with part 91 equipment) | Day/night VFR/IFR Congested and uncongested areas Class A, B, C, D, E and G airspace; (with part 91 equipment) Flight training and rental Towing | Day/night VFR/IFR Congested and uncongested areas Class A, B, C, D, E and G airspace; (with part 91 equipment) Flight training and rental Com- mercial operations |
| Maintenance | None | Owner- maintained | Maintenance-- Repairman (LS-M) Mechanic (A&P) Part 145 Repair station Preventive maintenance-- Sport pilot or higher | Owner-maintained | Maintenance-- Mechanic (A & P) Part 145 Repair station Preventive maintenance-- Recreation-al pilot or higher | Maintenance -- Mechanic (A & P) Part 145 Repair station Preventive maintenance - Recreation-al pilot or higher |
| Inspections | None (1) | Annual con- dition-- Repairman LS -I and LS-M Mechanic (A & P) Part 145 Repair station 100-hour condition (2)-- Repairman LS-M Mechanic (A & P) Part 145 Repair station | Annual condition-- Repairman LS - M Mechanic (A & P) Part 145 Repair station 100-hour condition (3)-- Repairman LS-M Mechanic (A & P) Part 145 Repair station | Annual condition-- Repairman (experimental aircraft builder) Mechanic (A & P) Part 145 repair station | Annual condition-- Mechanic (A & P) Part 145 repair station 100-hour condition (4)-- Mechanic (A & P) Part 145 Repair station | Annual condition-- Mechanic (A & P) Part 145 Repair station 100-hour condition (4)-- Mechanic (A & P) Part 145 repair station |
| Airworthiness Directives | None | None issued against ELSA | Yes - Type certificated TC/STC/PMA/TSO-approved products, if installed | None issued against amateur-built aircraft | Yes | Yes |
| Safety Directives | None | None | Yes | None | None | None |
| FAA Type or Product-ion Certificate | No | No | No | No | Yes | Yes |
| Consensus Standard | No | No (5) | Yes | No | No | No |

For two-place ultralight training vehicles operating under an exemption and registered with an FAA-recognized ultralight organization - 100-hour condition inspection done by ultralight instructor registered with an FAA-recognized ultralight organization.

- Applies to training aircraft used for compensation until January 31, 2010, and tow aircraft used for compensation

- Applies to aircraft used for flight training or towing for compensation

- Applies to aircraft used for flight instruction for hire - §91.409

- ELSA - Kit-built (§21.191(i)(2)(ii)) or aircraft that have been previously issued a special airworthiness certificate in the light-sport category (§21.191 (i)(3)) meet consensus standards